[image: image1.png]BAGGAGE LOADING anD TIE-DOWN

BAGGAGE AREA
MAXIMUM ALLOWABLE LOADS

AREA (D) =120 POUNDS
AREA (2) = 40 POUNDS

7/ AReas (D+ (@ =120 POUNDS

% TIE-DOWN NET ATTACH POINTS

% A tie-down net is provided to secure baggage in the baggage area.
The net attaches to six tie-down rings. Two rings are located on the floor
just aft of the seat backs and one ring is lgcated two inches above the floor
on each cabin wall at the aft end of area ® . Two additional rings are
located at the top, aft end of area . At least four rings should be used
to restrain the maximum baggage load of 120#.

n

If the airplane is equipped with utility shelf, it should be re-

moved prior to loading and tying down large baggage items. (Slide the tab of
the locking clips on each end of the shelf to disengage the shelf from the
aircraft structure.) After baggage is loaded and secured, either stow the
shelf or, if space permits, install it for storing small articles.




