Hi Fred,  here is a C.D. of some of the photos and drawings of the Iolaire,  they are not in order `cos I am not too good with this thingy,  but I hope they will be of some interest to you?.  Don`t forget,  I am not an expert but merely an enthusiastic amateur with most of my knowledge coming from model aircraft,  did you ever see the film called  “ The Flight of the Phoenix” ?  There are loads more photos under the bed,  if you want more, I will dig them out for you. 

If I were to make another Iolaire there are some wee adjustments (mods.) I would incorporate :-

a) To avoid the big 12volt car battery in the nose, I would move the pilot forward by using a thicker foam seat,  3 or 4 inches to better locate the CofG.

b) I would not use plain bearings in the main wheels,

c) I would employ tube clamps outboard of nylon 66 canard bearings to locate sideways and possibly an extra torque lever from these clamps to `fail safe` the epoxy tubular joint.

d) I would make the rear fuselage lower fairing a one piece fibre glass cowl.

e) Separate pedals (or a separate crossover steering bar) for the nosewheel steering.

f) Try the 4 belt reduction gear (lighter than the 5 speed modified BMW box) or the centrifugal clutch. The belts would allow a bit of slip when starting from cold, the BMW does not like props. when cold.

g) Angle the wing strut attachment point anchors to line up with strut angle ( as in the Quaiche ).

Hughie……………………..enjoy.

