### Viking Specifications

| #### VIKING AIRCRAFT ENGINE |  |
| --- | --- |
| Engine Type | Inline 4 cylinder |
| Displacement (cc/cu-in) | 1,497cc (91.4 cu-in) |
| Length | 619 mm (24.5 in) |
| Width | 559 mm (22 in) |
| Height | 450 mm (18 in) |
| Cooling | Liquid |
| Bore and Stroke (mm) | 73 x 90 |
| Fuel | Multi-Point Fuel Injection |
| Compression ratio | 10.4 |
| Horsepower (hp @ rpm) | 110 @ 5,500 (2,400 prop) (6.9 gph) |
| 75% Power (hp @ rpm) | 83 @ 4,200 (1,800 prop) (5.4 gph) |
| Torque @ prop (lb-ft @ rpm) | 247 @ 4,800 (2100 prop) |
| Dry Weight (lb) | 178 |
| Fuel Type | 92 Octane minimum (10% Ethanol max) You can use 100LL when you must |
| Engine Oil | 5W-20 Mobil 1 Synthetic |
| Gearbox Oil | 75W-90 Mobil 1 Synthetic gear oil |
| Cleanup | Do not wash engine. Wipe with WD-40 |

| PARAMETER | MINIMUM | MAXIMUM |
| --- | --- | --- |
| RPM | 1700 | 5840 |
| COOLANT TEMP (F) | NOT SPECIFIED DUE TO FAST WARMUP SYSTEM | 220 |
| OIL TEMPERATURE (F) | NOT SPECIFIED DUE TO FAST WARMUP SYSTEM | 220 |
| GEARBOX TEMPERATURE (F) | NOT SPECIFIED DUE TO FAST WARMUP SYSTEM | 220 |
| OIL PRESSURE (PSI) | 20 BELOW 4000 RPM / 45 ABOVE 4000 RPM | 100 |
| COOLANT PRESSURE (PSI) | THE ENGINE HAS A PRESSURELESS SYSTEM |  |
| FUEL PRESSURE | 39 | 47 |
| VOLTAGE | 13 | 15 |
| AMPERAGE | 3 - MEASURED ON ALTERNATOR CABLE | 40 |
| FUEL FLOW (GPH) | NOT SPECIFIED | 15 |